Network Security

Firewall

- What are some Benefits and Limitations of using Firewall in network security?

Benefits:

- Prevent exposure of sensitive hosts and applications to untrusted users

- The protocol flow can be sanitized, preventing the exploitation of protocol flaws

- Malicious data can be blocked from servers and clients

- Security policy enforcement can be made simple, scalable and robust with a properly configured firewall

Limitations:

- If misconfigured, a firewall can have serious consequences, such as single point of failure

- Network performance can slow down

- Intended data from many applications cannot be passed over firewalls securely

- Users might search for ways to bypass the firewall to receive blocked material e.g. connecting to mobile hotspot

- Unauthorized traffic can be tunnelled or hidden as legitimate traffic through the firewall

- What are the 3 types of Firewall in Network Security?  Which OSI model do each type of firewall work on?

- Packet filtering works on layers 3 & 4 of the osi model

- Stateful packet inspection works on layers 3,4 and 5 of the OSI model

- Proxy services (application-level gateways) work on layers 3,4,5 & 7 of the OSI model

- What are the characteristics of NGFW (Next Generation Firewall) ?

- Application awareness:

  - Does not assume a specific application is running on a specific port. Firewall can monitor traffic from layers 2 to 7 with greater granularity EG: http port 80 assumed to be http traffic. Useful for bandwidth control (P2P)

- Identity awareness:

  - Track identity of the local traffic device and user, typically using existing enterprise authentication systems (i.e active directory, LDAP). Control what a specific user or groups is allowed to send and receive.

- Extra intelligence

  - Optimized rule set and intelligence gathered from outside sources continually (whitelist, blacklist, directory integration to block by identity)

- Integrated IPS

  - Automatic correlation to IPS (To cover in PM) to suggest blocking of certain malicious websites. EG: block and address that is continually loading the IPS with bad traffic

- What are the three zones that are typically created in any firewall architecture?

- List the Industry best practices of deploying the firewalls.

- Deny all traffic by default, and only enable those services that are needed.

- Disable or uninstall any unnecessary services and software on the firewall that are not specifically required

- Limit the number of applications that run on the firewall in order to let the firewall do what it’s best at doing

- Run the firewall service as a unique user ID instead of administrator or root.

- Change the default firewall administrator or root password

- Do not rely on packet filtering alone. Use stateful inspection and application proxies if possible

- Ensure that physical access to the firewall is controlled

- Regularly monitor firewalls logs.

- Document all firewall rule changes.