# Network Assignment Report

# Executive Summary

This comprehensive report details a secure, and scalable network infrastructure for a multi-site

Enterprise (HQ Singapore, Jakarta/Tokyo branches, DMZ/Intranet) using VLSM IP design,

OSPF routing with MD5 authentication, IPSec VPNs, AAA/RADIUS central auth, NTP sync,

port-security, and granular firewall ACLs - with the achievement of CIA triad compliance with

minimal IP address wastage and defence-in-depth. All requirements function seamlessly via

the Cisco Packet Tracer Simulation, verified by DNS resolution to accounting.qwerty.com

across sites.

VLSM was used over FLSM to allocate variable subnet sizes matching host needs and

minimizing any waste of Ip address (e.g /28 for 14 hosts in HQ LAN vs /25 for larger Jakarta

LAN). Private IP addresses all range in the class C in order to enable efficient carving; public

/29 for WAN links support 6 hosts each (2 used for point-to-point). Sequential numbering aids

management; masks chosen as smallest power-of-2 exceeding required hosts +2

(network/broadcast addresses).

### Security Implementation Highlights:

Central Auth/Time: Usage of RADIUS + NTP on all routers; SSH-only through vty 0 1, no Telnet/Con/Aux.

Routing: OSPF Area 0 MD5 with a tuned hello/dead intervals for faster convergence and trust.

Encryption: IPSec site-to-site Tokyo-Jakarta (AES/SHA-HMAC, ACL 111 HTTPS to Jakarta Server, usage of vpnkey)

Access Control: Switch port-security and Firewall inbound ACLs for NTP, RADIUS, DNS, HTTP, HTTPS services for respective networks

Inspection: Stateful HTTP/ICMP deep checks; DNS records enable secure resolution

Usage of NAT for translation between Ip addresses in the site of connection to the internet (HQ

Router)

### Key Benefits & Verification

Includes Least-privilege blocks spoofing/DoS (no use of any ip any any), IPSec protects sensitive HTTPS over WAN. Verification of browser access to accounting.qwerty.com from branches/HQ; usage of OSPF neighbours full.

# Network IP addressing and FLSM/VLSM Design

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| HQ_Firewall H | Gig1/4 | 192.168.1.1/28 | 255.255.255.240 | NA |
|  | Gig1/3 | 192.168.2.1/29 | 255.255.255.248 | NA |
|  | Gig1/2 | 192.168.3.1/27 | 255.255.255.224 | NA |
|  | Gig1/1 | 209.165.200.225/29 | 255.255.255.248 | NA |
| Singapore (HQ) Router | FA0/0 | 209.165.200.226/29 | 255.255.255.248 | NA |
|  | Se0/0/0 | 192.168.4.1/30 | 255.255.255.252 | NA |
|  | Se0/0/1 | 192.168.5.1/30 | 255.255.255.252 | NA |
|  | Se0/1/0 | 209.165.200.2 | 255.255.255.248 | NA |
| Server-PT www.qwerty.com | Fa0 | 192.168.2.2/29 | 255.255.255.248 | 192.168.2.1 |
| Server-PT dns.qwerty.com | Fa0 | 192.168.2.3/29 | 255.255.255.248 | 192.168.2.1 |
| PC-PT HQ_Intranet_User | Fa0 | 192.168.3.2/27 | 255.255.255.224 | 192.168.3.1 |
| Server-PT NTP Server | Fa0 | 192.168.1.2/28 | 255.255.255.240 | 192.168.1.1 |
| Server-PT Accounting.qwerty.com | Fa0 | 192.168.1.3/28 | 255.255.255.240 | 192.168.1.1 |
| Server-PT RADIUS Server | Fa0 | 192.168.1.4/28 | 255.255.255.240 | 192.168.1.1 |
| Jakarta (Branch) | Se0/0/0 | 192.168.4.2/30 | 255.255.255.252 | NA |
|  | Fa0/0 | 192.168.6.1/25 | 255.255.255.128 | NA |
|  | Fa0/1 | 192.168.6.129/25 | 255.255.255.128 | NA |
| Server-PT Jakarta.qwerty.com | Fa0 | 192.168.6.130/25 | 255.255.255.128 | 192.168.6.129 |
| Laptop-PT Jakarta Laptop | Fa0 | 192.168.6.2/25 | 255.255.255.128 | 192.168.6.1 |
| Tokyo (Branch) | Se0/0/1 | 192.168.5.2/30 | 255.255.255.252 | NA |
|  | Fa0/1 | 192.168.7.1/29 | 255.255.255.248 | NA |
| Laptop-PT Tokyo Laptop | Fa0 | 192.168.7.2/29 | 255.255.255.248 | 192.168.7.1 |
| SP Router | Se0/1/0 | 209.165.200.1 | 255.255.255.248 | NA |
|  | Fa0/1 | 209.165.200.236 | 255.255.255.248 | NA |
| Server-PT www.internet.com | Fa0/2 | 209.165.200.234 | 255.255.255.248 | 209.165.200.236 |
| Laptop-PT Internet User | Fa0/24 | 209.165.200.235 | 255.255.255.248 | 209.165.200.236 |

# All Devices:

Enable password : 242100Q_enable

Service password-encryption: Enabled

Shutdown all unused ports

All routers, switches, firewalls are named to the requirements of 242100Q_device(location)

# For Tokyo Router,

## NTP Time-Synchronize and password encryption

NTP Time-Synchronization ntp authentication-key 1 md5 SITNetSec ntp authenticate ntp trusted-key 1 ntp server 192.168.1.2 ntp update-calendar Servicing of Password-Encryption service password-encryption

- Configured to prevent any spoofing, tampering and Man-in-the-Middle (MitM) attacks.

- Ensures Accurate time, critical for security compliance and using authentication key for trusted NTP sources

## AAA through Radius server

Configuration of Radius server and AAA Enable Config t Username Tokyo_Admin privilege 15 secret 242100Q Ip domain-name cisco.com Aaa new-model Radius-server host 192.168.1.4 auth-port 1645 Radius-server key 242100Q aaa authentication login default group radius local crypto key generate rsa 1024 SSH connection and disable other connection (Aux, Con, Tel) line vty 0 1 transport input ssh login authentication default line vty 2 15 transport input none line con 0 exec-timeout 0 0 line aux 0 exec-timeout 0 0 exit

AAA used for centralization of authentication across routers(HQ, Jakarta, Tokyo); Ensuring:

- Verification of user identities and determines authorized configuration of network services

- Configuration of SSH connection only through vty 0 1 to limit and prevent any unauthorized connection through other protocols (e.g telnet, auxiliary, console)

## OSPF Routing and Hello and Dead intervals

Router ospf 1 area 0 authentication message-digest network 192.168.5.0 0.0.0.3 area 0 network 192.168.7.0 0.0.0.7 area 0 interface serial 0/0/1 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf hello-interval 10 ip ospf dead-interval 15 interface fa0/1 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf hello-interval 10 ip ospf dead-interval 15

OSPF MD5 authentication using key (242100Q_md5) and configuring hello and dead intervals

of 10 and 15 respectively to:

- Authentication Type 2 to ensure secure OSPF routing exchanges and only trusted routers can participate in the network

- Similar dead and hello intervals for maintaining neighbour relationships and ensuring quick network convergence

## IPSec site-to-site with Jakarta

| Parameters | Tokyo | Jakarta |
| --- | --- | --- |
| Transform Set Name | VPN-SET | VPN-SET |
| ESP Transform Encryption | esp-aes | esp-aes |
| ESP Transform Authentication | esp-sha-hmac | esp-sha-hmac |
| Peer IP Address | 192.168.4.2 | 192.168.5.2 |
| Traffic to be Encrypted | access-list 111 (source 192.168.7.0 dest 192.168.6.0) | access-list 111 (source 192.168.6.0 dest 192.168.7.0) |
| Crypto Map Name | VPN-MAP | VPN-MAP |
| SA Establishment | ipsec-isakmp | ipsec-isakmp |

With the diagram at the top for referencing the IPSec parameters used to configure IPSec direct connectivity between Tokyo and Jakarta.

Access list used to permit any HTTPS traffic sent by Tokyo subnet as interesting traffic access-list 111 permit tcp 192.168.7.0 0.0.0.7 host 192.168.6.130 eq 443 Creation of ISAKMP Policy crypto isakmp policy 10 encryption aes hash sha authentication pre-share group 2 exit Setting Pre-shared key crypto isakmp key vpnkey address 192.168.4.2 Creation of Transform Set crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac Create and Map the Crypto Map crypto map VPN-MAP 10 ipsec-isakmp set peer 192.168.4.2 set transform-set VPN-SET match address 111 exit Applying Map to Interface facing connection interface serial 0/0/1 crypto map VPN-MAP

The IPSec VPN Site-to-Site Implementation between Tokyo and Jakarta Router to encrypt

traffic going to the Jakarta Server over HTTPS only.

- This ensures that this connection is able to connect securely over the public internet without the internet users being able to connect to it over HTTPS.

- It also ensures that the HTTPS request is protected as sensitive data so that it will not be intercepted nor tampered during the transmission through cryptographic services.

## Port Security on Switch 4

Configuring maximum of 3 MAC addresses connected to Switch Enable Config t Interface range fa 0/1-2 Switchport port-security maximum 3 Exit Shutdown all unused ports on switch Interface range fa 0/3-24 Shutdown Interface range gig 0/1-2 Shutdown Interface vlan 1 Shutdown

Configuration of switch to ensure:

- Maximum number of MAC addresses (end devices) to connect to the switch are limited to three

- Unused ports /interfaces are shutdown to prevent any unauthorized connection to the network

# For Jakarta Router

## NTP Time-Synchronize and password encryption

NTP Time-Synchronization ntp authentication-key 1 md5 SITNetSec ntp authenticate ntp trusted-key 1 ntp server 192.168.1.2 ntp update-calendar Servicing of Password-Encryption service password-encryption

- Configured to prevent any spoofing, tampering and Man-in-the-Middle (MitM) attacks.

- Ensures Accurate time, critical for security compliance and using authentication key for trusted NTP sources

## AAA through Radius Server

Configuration of Radius server and AAA Enable Config t Username Jakarta_Admin privilege 15 secret 242100Q Ip domain-name cisco.com Aaa new-model Radius-server host 192.168.1.4 auth-port 1645 Radius-server key 242100Q aaa authentication login default group radius local crypto key generate rsa 1024 SSH connection and disable other connection (Aux, Con, Tel) line vty 0 1 transport input ssh login authentication default line vty 2 15 transport input none line con 0 exec-timeout 0 0 line aux 0 exec-timeout 0 0 exit

AAA used for centralization of authentication across routers(HQ, Jakarta, Tokyo); Ensuring:

- Verification of user identities and determines authorized configuration of network services

- Configuration of SSH connection only through vty 0 1 to limit and prevent any unauthorized connection through other protocols (e.g telnet, auxiliary, console)

## OSPF Routing and Hello and Dead Intervals

Router ospf 1 area 0 authentication message-digest network 192.168.4.0 0.0.0.3 area 0 network 192.168.6.0 0.0.0.127 area 0 network 192.168.6.128 0.0.0.127 area 0 interface serial 0/0/0 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf hello-interval 10 ip ospf dead-interval 15 interface fa0/0 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf hello-interval 10 ip ospf dead-interval 15 interface fa0/1 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf hello-interval 10 ip ospf dead-interval 15

OSPF MD5 authentication using key (242100Q_md5) and configuring hello and dead intervals

of 10 and 15 respectively to:

- Authentication Type 2 to ensure secure OSPF routing exchanges and only trusted routers can participate in the network

- Similar dead and hello intervals for maintaining neighbour relationships and ensuring quick network convergence

## IPSec site-to-site with Tokyo

Mirror ACL from reverse destination (Tokyo) access-list 111 permit tcp host 192.168.6.130 eq 443 192.168.7.0 0.0.0.7 Phase 1 Policy (Must match Tokyo) crypto isakmp policy 10 encryption aes hash sha authentication pre-share group 2 exit Set the Pre-Shared Key crypto isakmp key vpnkey address 192.168.5.2 Define Phase 2 (Transform Set) crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac Create Crypto Map crypto map VPN-MAP 10 ipsec-isakmp set peer 192.168.5.2 set transform-set VPN-SET match address 111 exit Apply to the Outgoing Interface (facing the other IPSec implementation) interface Serial 0/0/0 crypto map VPN-MAP

The IPSec VPN Site-to-Site Implementation between Tokyo and Jakarta Router to encrypt

traffic going to the Jakarta Server over HTTPS only.

- This ensures that this connection is able to connect securely over the public internet without the internet users being able to connect to it over HTTPS.

- It also ensures that the HTTPS request is protected as sensitive data so that it will not be intercepted nor tampered during the transmission through cryptographic services.

## Port Security on Switch 3

Configuring maximum of 2 MAC addresses connected to Switch Enable Config t Interface range fa 0/1-2 Switchport port-security maximum 2 Exit Shutdown all unused ports on switch Interface range fa 0/3-24 Shutdown Interface range gig 0/1-2 Shutdown Interface vlan 1 Shutdown

Configuration of switch to ensure:

- Maximum number of MAC addresses (end devices) to connect to the switch are limited to two

- Unused ports /interfaces are shutdown to prevent any unauthorized connection to the network

# HQ @Singapore Router

## NTP Time-Synchronize and password encryption

NTP Time-Synchronization ntp authentication-key 1 md5 SITNetSec ntp authenticate ntp trusted-key 1 ntp server 192.168.1.2 ntp update-calendar Servicing of Password-Encryption service password-encryption

- Configured to prevent any spoofing, tampering and Man-in-the-Middle (MitM) attacks.

- Ensures Accurate time, critical for security compliance and using authentication key for trusted NTP sources

## AAA through Radius Server

Configuration of Radius server and AAA Enable Config t Username HQ_Admin privilege 15 secret 242100Q Ip domain-name cisco.com Aaa new-model Radius-server host 192.168.1.4 auth-port 1645 Radius-server key 242100Q aaa authentication login default group radius local crypto key generate rsa 1024 SSH connection and disable other connection (Aux, Con, Tel) line vty 0 1 transport input ssh login authentication default line vty 2 15 transport input none line con 0 exec-timeout 0 0 line aux 0 exec-timeout 0 0 exit

AAA used for centralization of authentication across routers(HQ, Jakarta, Tokyo); Ensuring:

- Verification of user identities and determines authorized configuration of network services

- Configuration of SSH connection only through vty 0 1 to limit and prevent any unauthorized connection through other protocols (e.g telnet, auxiliary, console)

## OSPF Routing, Default routing and Hello and Dead Intervals

ip route 0.0.0.0 0.0.0.0 Serial0/1/0 Router ospf 1 area 0 authentication message-digest network 192.168.4.0 0.0.0.3 area 0 network 192.168.5.0 0.0.0.3 area 0 network 209.165.200.224 0.0.0.7 area 0 default-information originate interface s 0/0/0 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf hello-interval 10 ip ospf dead-interval 15 interface s0/0/1 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf hello-interval 10 ip ospf dead-interval 15 interface fa0/0 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf hello-interval 10 ip ospf dead-interval 15

OSPF MD5 authentication using key (242100Q_md5) and configuring hello and dead intervals

of 10 and 15 respectively to:

- Authentication Type 2 to ensure secure OSPF routing exchanges and only trusted routers can participate in the network

- Similar dead and hello intervals for maintaining neighbour relationships and ensuring quick network convergence

Static Routing towards the Internet Subnet to ensure:

- Saving CPU and bandwidth

- Specifying the gateway to ensure the internet connection only through this link for all networks

## NAT Configuration

Ip access-list standard INSIDE-LOCAL permit 192.168.3.0 0.0.0.31 ip nat inside source static tcp 192.168.2.2 80 209.165.200.2 80 ip nat inside source static tcp 192.168.2.2 443 209.165.200.2 443 ip nat inside source static udp 192.168.2.3 53 209.165.200.2 53 ip nat inside source list INSIDE-LOCAL interface Serial0/1/0 overload int fa 0/0 ip nat inside int s0/1/0 ip nat outside exit

Used to configure static NAT to allow consistent access to the specific subnet servers (DMZ)

Through the translation of public subnetting in the same VLSM for better security and better access.

Used NAT overload to allow inside connection (Intranet user) to the outside server (Internet.com) for private IP translation to public IP translation.

# For ISP Router

## Routing

Default routing to the internet subnet Enable Config t ip route 0.0.0.0 0.0.0.0 FastEthernet0/1

Configuration of a static routing:

- To ensure that a gateway to the internet subnet through the router is created and the only path to the network with no restrictions

# For Firewall

## OSPF Routing and hello and dead interval

Router ospf 1 area 0 authentication message-digest network 209.165.200.224 255.255.255.248 area 0 network 192.168.1.0 255.255.255.240 area 0 network 192.168.2.0 255.255.255.248 area 0 network 192.168.3.0 255.255.255.224 area 0 interface gig 1/1 ospf message-digest-key 1 md5 242100Q_md5 ospf hello-interval 10 ospf dead-interval 15

OSPF MD5 authentication using key (242100Q_md5) and configuring hello and dead intervals

of 10 and 15 respectively to:

- Authentication Type 2 to ensure secure OSPF routing exchanges and only trusted routers can participate in the network

- Similar dead and hello intervals for maintaining neighbour relationships and ensuring quick network convergence

## ACLs Implemented on firewall

Enable Config t Tokyo ACLs NTP Service Access-list ALL-IN Permit udp 192.168.5.0 255.255.255.252 host 192.168.1.2 eq 123 AAA Service Access-list ALL-IN Permit udp 192.168.5.0 255.255.255.252 host 192.168.1.4 eq 1645 Access-list ALL-IN Permit udp 192.168.5.0 255.255.255.252 host 192.168.1.4 eq 1646 DNS, HTTPS, and HTTP Service Access-list ALL-IN Permit udp 192.168.7.0 255.255.255.248 any eq 53 Access-list ALL-IN Permit tcp 192.168.7.0 255.255.255.248 any eq 443 Access-list ALL-IN Permit tcp 192.168.7.0 255.255.255.248 any eq 80 Jakarta ACLs NTP Service Access-list ALL-IN Permit udp 192.168.4.0 255.255.255.252 host 192.168.1.2 eq 123 AAA Service Access-list ALL-IN Permit udp 192.168.4.0 255.255.255.252 host 192.168.1.4 eq 1645 Access-list ALL-IN Permit udp 192.168.4.0 255.255.255.252 host 192.168.1.4 eq 1646 DNS, HTTPS, HTTP Service for Jakarta Subnet Access-list ALL-IN Permit udp 192.168.6.0 255.255.255.128 any eq 53 Access-list ALL-IN Permit tcp 192.168.6.0 255.255.255.128 any eq 443 Access-list ALL-IN Permit tcp 192.168.6.0 255.255.255.128 any eq 80 DNS, HTTPS Service for Intranet Subnet Access-list ALL-IN Permit udp 192.168.6.128 255.255.255.128 any eq 53 access-list ALL-IN extended permit tcp host 192.168.6.130 eq 443 192.168.3.0 255.255.255.224 HQ@Singapore ACLs NTP Service access-list ALL-IN extended permit udp 209.165.200.224 255.255.255.248 host 192.168.1.2 eq 123 AAA Service access-list ALL-IN extended permit udp 209.165.200.224 255.255.255.248 host 192.168.1.4 eq 1645 access-list ALL-IN extended permit udp 209.165.200.224 255.255.255.248 host 192.168.1.4 eq 1646 Internet ACLs Permit Access to only HTTP, DNS, HTTPS to DMZ Area(Web server, DNS server) only access-list ALL-IN extended permit udp 209.165.200.232 255.255.255.248 192.168.2.0 255.255.255.248 eq domain access-list ALL-IN extended permit tcp 209.165.200.232 255.255.255.248 192.168.2.0 255.255.255.248 eq www access-list ALL-IN extended permit tcp 209.165.200.232 255.255.255.248 192.168.2.0 255.255.255.248 eq 443 Permit HTTPS back into Intranet Users access-list ALL-IN extended permit tcp host 192.168.6.130 eq 443 192.168.3.0 255.255.255.224 Apply ACLs on interface outside to allow one way in and out methods Access-group ALL-IN in interface outside

All on one ACLs to permit through one interface only instead of multiple to prevent over complication of activity

## Additional Security Implementations to Firewall

Provide Firewall ability to inspect http and https pings made by server Enable Config t Class inspection-default Inspect http Inspect icmp

- Inspection of HTTP/HTTPS and ICMP packets to ensure deep packet checks and allow permission for HTTP and HTTPS

# DMZ Subnet

## Verification  access to HTTP and HTTPS of web access

Ensure end-devices (PC, Laptops) have a DNS server gateway (192.168.2.3).

In the DNS server (192.168.2.3), ensure to add account.qwerty.com (192.168.1.3) as “record

A” to the DNS records in the server services. To allow NAT public address to access DMZ web server, the public address added in as well for access.

To verify results, go to any end device in the any subnet (Tokyo, Jakarta, Intranet)and enter the

web browser in desktop tab “accounting.qwerty.com” and the following page should show up.

# Server Farm

## Radius Server

On the Radius Server services of AAA, ensuring to put in all client information (router

information) and the user set-up of “Radius_Admin” with password as “242100Q”.

- Enable authentication of routers using the AAA through Radius_Server

## NTP Server

On the NTP Server on the services of NTP, ensure to “On” NTP and Enable Authentication

and enter in the respective values (Key = 1, Password = SITNetSec) to ensure that routers with

the authentication key 1 with password of SITNetSec can be authenticated over the NTP

service.

# Assumptions

Jakarta Subnetting shares with the HQ subnet users, hence /25 to accommodate for the HQ

users + Jakarta users.

Simulated environment: NAT already configured for the translation of private to public

addressing

# Annex

## Tokyo Router

Current configuration : 2106 bytes ! version 12.4 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_Tokyo ! ! ! enable secret 5 $1$mERr$7QVi9O/SygRnlYHzfjQbC. ! ! ! ! ! aaa new-model ! aaa authentication login default group radius local ! ! ! ! ! ! ! ip cef no ipv6 cef ! ! ! username Tokyo_Admin privilege 15 secret 5 $1$mERr$qIuQAvEBEHQIpDl3eqvYs. ! ! crypto isakmp policy 10 encr aes 256 authentication pre-share group 2 ! crypto isakmp key 242100Q address 192.168.4.2 ! ! ! crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac ! crypto map VPN-MAP 10 ipsec-isakmp set peer 192.168.4.2 set transform-set VPN-SET match address 111 ! ! ! ! ip domain-name cisco.com ! ! spanning-tree mode pvst ! ! ! ! ! ! interface FastEthernet0/0 no ip address duplex auto speed auto shutdown ! interface FastEthernet0/1 ip address 192.168.7.1 255.255.255.248 ip ospf message-digest-key 1 md5 242100Q_md5 duplex auto speed auto ! interface Serial0/0/0 no ip address clock rate 2000000 shutdown ! interface Serial0/0/1 ip address 192.168.5.2 255.255.255.252 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf dead-interval 15 clock rate 2000000 crypto map VPN-MAP ! interface Serial0/1/0 no ip address clock rate 2000000 shutdown ! interface Serial0/1/1 no ip address clock rate 2000000 shutdown ! interface Vlan1 no ip address shutdown ! router ospf 1 log-adjacency-changes area 0 authentication message-digest network 192.168.5.0 0.0.0.3 area 0 network 192.168.7.0 0.0.0.7 area 0 ! ip classless ! ip flow-export version 9 ! ! access-list 111 permit tcp 192.168.7.0 0.0.0.7 host 192.168.6.130 eq 443 ! no cdp run ! ! radius-server host 192.168.1.4 auth-port 1645 radius-server key 242100Q ! ! ! ! line con 0 exec-timeout 0 0 ! line aux 0 exec-timeout 0 0 ! line vty 0 1 login authentication default transport input ssh line vty 2 4 transport input none line vty 5 15 transport input none ! ! ntp authentication-key 1 md5 0812657A271C11241708 7 ntp authenticate ntp trusted-key 1 ntp server 192.168.1.2 ntp update-calendar ! end

## Jakarta Router

Current configuration : 2172 bytes ! version 12.4 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_Jakarta ! ! ! enable secret 5 $1$mERr$7QVi9O/SygRnlYHzfjQbC. ! ! ! ! ! aaa new-model ! aaa authentication login default group radius local ! ! ! ! ! ! ! no ip cef no ipv6 cef ! ! ! username Jakarta_Admin privilege 15 secret 5 $1$mERr$qIuQAvEBEHQIpDl3eqvYs. ! ! crypto isakmp policy 10 encr aes 256 authentication pre-share group 2 ! crypto isakmp key 242100Q address 192.168.5.2 ! ! ! crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac ! crypto map VPN-MAP 10 ipsec-isakmp set peer 192.168.5.2 set transform-set VPN-SET match address 111 ! ! ! ! ip domain-name cisco.com ! ! spanning-tree mode pvst ! ! ! ! ! ! interface FastEthernet0/0 ip address 192.168.6.1 255.255.255.128 duplex auto speed auto ! interface FastEthernet0/1 ip address 192.168.6.129 255.255.255.128 ip ospf message-digest-key 1 md5 242100Q_md5 duplex auto speed auto ! interface Serial0/0/0 ip address 192.168.4.2 255.255.255.252 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf dead-interval 15 clock rate 2000000 crypto map VPN-MAP ! interface Serial0/0/1 no ip address clock rate 2000000 shutdown ! interface Serial0/1/0 no ip address clock rate 2000000 shutdown ! interface Serial0/1/1 no ip address clock rate 2000000 shutdown ! interface Vlan1 no ip address shutdown ! router ospf 1 log-adjacency-changes area 0 authentication message-digest network 192.168.4.0 0.0.0.3 area 0 network 192.168.6.0 0.0.0.127 area 0 network 192.168.6.128 0.0.0.127 area 0 ! ip classless ! ip flow-export version 9 ! ! access-list 111 permit tcp host 192.168.6.130 eq 443 192.168.7.0 0.0.0.7 ! no cdp run ! ! radius-server host 192.168.1.4 auth-port 1645 radius-server key 242100Q ! ! ! ! line con 0 exec-timeout 0 0 ! line aux 0 exec-timeout 0 0 ! line vty 0 1 login authentication default transport input ssh line vty 2 4 transport input none line vty 5 15 transport input none ! ! ntp authentication-key 1 md5 0812657A271C11241708 7 ntp authenticate ntp trusted-key 1 ntp server 192.168.1.2 ntp update-calendar ! end

## HQ@Singapore Router

Current configuration : 2262 bytes ! version 12.4 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_HQ ! ! ! enable secret 5 $1$mERr$7QVi9O/SygRnlYHzfjQbC. ! ! ! ! ! aaa new-model ! aaa authentication login default group radius local ! ! ! ! ! ! ! ip cef no ipv6 cef ! ! ! username HQ_Admin privilege 15 secret 5 $1$mERr$qIuQAvEBEHQIpDl3eqvYs. ! ! ! ! ! ! ! ! ip domain-name cisco.com ! ! spanning-tree mode pvst ! ! ! ! ! ! interface FastEthernet0/0 ip address 209.165.200.226 255.255.255.248 ip ospf message-digest-key 1 md5 242100Q_md5 ip nat inside duplex auto speed auto ! interface FastEthernet0/1 no ip address duplex auto speed auto shutdown ! interface Serial0/0/0 ip address 192.168.4.1 255.255.255.252 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf dead-interval 15 ! interface Serial0/0/1 ip address 192.168.5.1 255.255.255.252 ip ospf message-digest-key 1 md5 242100Q_md5 ip ospf dead-interval 15 ! interface Serial0/1/0 ip address 209.165.200.2 255.255.255.248 ip nat outside clock rate 2000000 ! interface Serial0/1/1 no ip address clock rate 2000000 shutdown ! interface Vlan1 no ip address shutdown ! router ospf 1 log-adjacency-changes area 0 authentication message-digest network 192.168.4.0 0.0.0.3 area 0 network 192.168.5.0 0.0.0.3 area 0 network 209.165.200.224 0.0.0.7 area 0 default-information originate ! ip nat inside source list INSIDE-LOCAL interface Serial0/1/0 overload ip nat inside source static tcp 192.168.2.2 80 209.165.200.2 80 ip nat inside source static tcp 192.168.2.2 443 209.165.200.2 443 ip nat inside source static udp 192.168.2.3 53 209.165.200.2 53 ip classless ip route 0.0.0.0 0.0.0.0 Serial0/1/0 ! ip flow-export version 9 ! ! ip access-list standard INSIDE-LOCAL permit 192.168.3.0 0.0.0.31 ! no cdp run ! ! radius-server host 192.168.1.4 auth-port 1645 radius-server key 242100Q ! ! ! ! line con 0 exec-timeout 0 0 ! line aux 0 exec-timeout 0 0 ! line vty 0 1 login authentication default transport input ssh line vty 2 4 transport input none line vty 5 15 transport input none ! ! ntp authentication-key 1 md5 0812657A271C11241708 7 ntp authenticate ntp trusted-key 1 ntp server 192.168.1.2 ntp update-calendar ! end

## ISP Router

Current configuration : 1073 bytes ! version 12.4 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_ISP ! ! ! enable password 7 0873181C584955262D0E0205282721 ! ! ! ! ! ! ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! spanning-tree mode pvst ! ! ! ! ! ! interface FastEthernet0/0 no ip address duplex auto speed auto shutdown ! interface FastEthernet0/1 ip address 209.165.200.236 255.255.255.248 duplex auto speed auto ! interface Serial0/0/0 no ip address clock rate 2000000 shutdown ! interface Serial0/0/1 no ip address clock rate 2000000 shutdown ! interface Serial0/1/0 ip address 209.165.200.1 255.255.255.248 ! interface Serial0/1/1 no ip address clock rate 2000000 shutdown ! interface Vlan1 no ip address shutdown ! router ospf 1 log-adjacency-changes default-information originate ! ip classless ip route 0.0.0.0 0.0.0.0 FastEthernet0/1 ! ip flow-export version 9 ! ! ! no cdp run ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end

## Firewall ASA 5506

ASA Version 9.6(1) ! hostname 242100QFirewall enable password lgwuysk3tYU3Risq encrypted names ! interface GigabitEthernet1/1 nameif outside security-level 0 ip address 209.165.200.225 255.255.255.248 ospf message-digest-key 1 md5 ***** ! interface GigabitEthernet1/2 nameif INTRANET security-level 100 ip address 192.168.3.1 255.255.255.224 ! interface GigabitEthernet1/3 nameif DMZ security-level 70 ip address 192.168.2.1 255.255.255.248 ! interface GigabitEthernet1/4 nameif INTRANET_SERVER_FARM security-level 100 ip address 192.168.1.1 255.255.255.240 ! interface GigabitEthernet1/5 no nameif no security-level no ip address shutdown ! interface GigabitEthernet1/6 no nameif no security-level no ip address shutdown ! interface GigabitEthernet1/7 no nameif no security-level no ip address shutdown ! interface GigabitEthernet1/8 no nameif no security-level no ip address shutdown ! interface Management1/1 management-only no nameif no security-level no ip address shutdown ! ! ! access-list ALL-IN extended permit udp 192.168.5.0 255.255.255.252 host 192.168.1.2 eq 123 access-list ALL-IN extended permit udp 192.168.5.0 255.255.255.252 host 192.168.1.4 eq 1645 access-list ALL-IN extended permit udp 192.168.5.0 255.255.255.252 host 192.168.1.4 eq 1646 access-list ALL-IN extended permit udp 192.168.7.0 255.255.255.248 any eq domain access-list ALL-IN extended permit tcp 192.168.7.0 255.255.255.248 any eq 443 access-list ALL-IN extended permit tcp 192.168.7.0 255.255.255.248 any eq www access-list ALL-IN extended permit udp 192.168.4.0 255.255.255.252 host 192.168.1.2 eq 123 access-list ALL-IN extended permit udp 192.168.4.0 255.255.255.252 host 192.168.1.4 eq 1645 access-list ALL-IN extended permit udp 192.168.4.0 255.255.255.252 host 192.168.1.4 eq 1646 access-list ALL-IN extended permit udp 192.168.6.0 255.255.255.128 any eq domain access-list ALL-IN extended permit tcp 192.168.6.0 255.255.255.128 any eq 443 access-list ALL-IN extended permit tcp 192.168.6.0 255.255.255.128 any eq www access-list ALL-IN extended permit udp 192.168.6.128 255.255.255.128 any eq domain access-list ALL-IN extended permit udp 209.165.200.224 255.255.255.248 host 192.168.1.2 eq 123 access-list ALL-IN extended permit udp 209.165.200.224 255.255.255.248 host 192.168.1.4 eq 1645 access-list ALL-IN extended permit udp 209.165.200.224 255.255.255.248 host 192.168.1.4 eq 1646 access-list ALL-IN extended permit udp 209.165.200.232 255.255.255.248 192.168.2.0 255.255.255.248 eq domain access-list ALL-IN extended permit tcp 209.165.200.232 255.255.255.248 192.168.2.0 255.255.255.248 eq www access-list ALL-IN extended permit tcp 209.165.200.232 255.255.255.248 192.168.2.0 255.255.255.248 eq 443 access-list ALL-IN extended permit tcp host 192.168.6.130 eq 443 192.168.3.0 255.255.255.224 ! ! access-group ALL-IN in interface outside ! ! class-map inspection_default match default-inspection-traffic class-map inspection-default match default-inspection-traffic ! policy-map type inspect dns preset_dns_map parameters message-length maximum 512 policy-map global_policy class inspection_default inspect dns preset_dns_map inspect ftp inspect tftp class inspection-default inspect http inspect icmp ! service-policy global_policy global ! telnet timeout 5 ssh timeout 5 ! ! ! ! router ospf 1 log-adjacency-changes area 0 authentication message-digest network 209.165.200.224 255.255.255.248 area 0 network 192.168.1.0 255.255.255.240 area 0 network 192.168.2.0 255.255.255.248 area 0 network 192.168.3.0 255.255.255.224 area 0 !

## DNS Server

## NTP Server

## RADIUS Server

## Switch 0

Current configuration : 1374 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_InternetSwitch ! enable password 7 0873181C584955262D0E0205282721 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! interface FastEthernet0/1 ! interface FastEthernet0/2 ! interface FastEthernet0/3 shutdown ! interface FastEthernet0/4 shutdown ! interface FastEthernet0/5 shutdown ! interface FastEthernet0/6 shutdown ! interface FastEthernet0/7 shutdown ! interface FastEthernet0/8 shutdown ! interface FastEthernet0/9 shutdown ! interface FastEthernet0/10 shutdown ! interface FastEthernet0/11 shutdown ! interface FastEthernet0/12 shutdown ! interface FastEthernet0/13 shutdown ! interface FastEthernet0/14 shutdown ! interface FastEthernet0/15 shutdown ! interface FastEthernet0/16 shutdown ! interface FastEthernet0/17 shutdown ! interface FastEthernet0/18 shutdown ! interface FastEthernet0/19 shutdown ! interface FastEthernet0/20 shutdown ! interface FastEthernet0/21 shutdown ! interface FastEthernet0/22 shutdown ! interface FastEthernet0/23 shutdown ! interface FastEthernet0/24 ! interface GigabitEthernet0/1 shutdown ! interface GigabitEthernet0/2 shutdown ! interface Vlan1 no ip address shutdown ! ! ! ! line con 0 ! line vty 0 4 login line vty 5 15 login ! ! ! ! end

## Switch 1 (Server Farm)

Current configuration : 1366 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_ServerFarmSwitch ! enable password 7 0873181C584955262D0E0205282721 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! interface FastEthernet0/1 ! interface FastEthernet0/2 ! interface FastEthernet0/3 ! interface FastEthernet0/4 ! interface FastEthernet0/5 shutdown ! interface FastEthernet0/6 shutdown ! interface FastEthernet0/7 shutdown ! interface FastEthernet0/8 shutdown ! interface FastEthernet0/9 shutdown ! interface FastEthernet0/10 shutdown ! interface FastEthernet0/11 shutdown ! interface FastEthernet0/12 shutdown ! interface FastEthernet0/13 shutdown ! interface FastEthernet0/14 shutdown ! interface FastEthernet0/15 shutdown ! interface FastEthernet0/16 shutdown ! interface FastEthernet0/17 shutdown ! interface FastEthernet0/18 shutdown ! interface FastEthernet0/19 shutdown ! interface FastEthernet0/20 shutdown ! interface FastEthernet0/21 shutdown ! interface FastEthernet0/22 shutdown ! interface FastEthernet0/23 shutdown ! interface FastEthernet0/24 shutdown ! interface GigabitEthernet0/1 shutdown ! interface GigabitEthernet0/2 shutdown ! interface Vlan1 no ip address shutdown ! ! ! ! line con 0 ! line vty 0 4 login line vty 5 15 login ! ! ! ! end

## Switch 1 (DMZ)

Current configuration : 1369 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_DMZSwitch ! enable password 7 0873181C584955262D0E0205282721 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! interface FastEthernet0/1 ! interface FastEthernet0/2 ! interface FastEthernet0/3 ! interface FastEthernet0/4 shutdown ! interface FastEthernet0/5 shutdown ! interface FastEthernet0/6 shutdown ! interface FastEthernet0/7 shutdown ! interface FastEthernet0/8 shutdown ! interface FastEthernet0/9 shutdown ! interface FastEthernet0/10 shutdown ! interface FastEthernet0/11 shutdown ! interface FastEthernet0/12 shutdown ! interface FastEthernet0/13 shutdown ! interface FastEthernet0/14 shutdown ! interface FastEthernet0/15 shutdown ! interface FastEthernet0/16 shutdown ! interface FastEthernet0/17 shutdown ! interface FastEthernet0/18 shutdown ! interface FastEthernet0/19 shutdown ! interface FastEthernet0/20 shutdown ! interface FastEthernet0/21 shutdown ! interface FastEthernet0/22 shutdown ! interface FastEthernet0/23 shutdown ! interface FastEthernet0/24 shutdown ! interface GigabitEthernet0/1 shutdown ! interface GigabitEthernet0/2 shutdown ! interface Vlan1 no ip address shutdown ! ! ! ! line con 0 ! line vty 0 4 login line vty 5 15 login ! ! ! ! end

## Switch 2

Current configuration : 1384 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_IntranetSwitch ! enable password 7 0873181C584955262D0E0205282721 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! interface FastEthernet0/1 ! interface FastEthernet0/2 ! interface FastEthernet0/3 shutdown ! interface FastEthernet0/4 shutdown ! interface FastEthernet0/5 shutdown ! interface FastEthernet0/6 shutdown ! interface FastEthernet0/7 shutdown ! interface FastEthernet0/8 shutdown ! interface FastEthernet0/9 shutdown ! interface FastEthernet0/10 shutdown ! interface FastEthernet0/11 shutdown ! interface FastEthernet0/12 shutdown ! interface FastEthernet0/13 shutdown ! interface FastEthernet0/14 shutdown ! interface FastEthernet0/15 shutdown ! interface FastEthernet0/16 shutdown ! interface FastEthernet0/17 shutdown ! interface FastEthernet0/18 shutdown ! interface FastEthernet0/19 shutdown ! interface FastEthernet0/20 shutdown ! interface FastEthernet0/21 shutdown ! interface FastEthernet0/22 shutdown ! interface FastEthernet0/23 shutdown ! interface FastEthernet0/24 shutdown ! interface GigabitEthernet0/1 shutdown ! interface GigabitEthernet0/2 shutdown ! interface Vlan1 no ip address shutdown ! ! ! ! line con 0 ! line vty 0 4 login line vty 5 15 login ! ! ! ! end

## Switch 3

Current configuration : 1455 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_JakartaSwitch ! enable password 7 0873181C584955262D0E0205282721 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! interface FastEthernet0/1 switchport port-security maximum 2 ! interface FastEthernet0/2 switchport port-security maximum 2 ! interface FastEthernet0/3 shutdown ! interface FastEthernet0/4 shutdown ! interface FastEthernet0/5 shutdown ! interface FastEthernet0/6 shutdown ! interface FastEthernet0/7 shutdown ! interface FastEthernet0/8 shutdown ! interface FastEthernet0/9 shutdown ! interface FastEthernet0/10 shutdown ! interface FastEthernet0/11 shutdown ! interface FastEthernet0/12 shutdown ! interface FastEthernet0/13 shutdown ! interface FastEthernet0/14 shutdown ! interface FastEthernet0/15 shutdown ! interface FastEthernet0/16 shutdown ! interface FastEthernet0/17 shutdown ! interface FastEthernet0/18 shutdown ! interface FastEthernet0/19 shutdown ! interface FastEthernet0/20 shutdown ! interface FastEthernet0/21 shutdown ! interface FastEthernet0/22 shutdown ! interface FastEthernet0/23 shutdown ! interface FastEthernet0/24 shutdown ! interface GigabitEthernet0/1 shutdown ! interface GigabitEthernet0/2 shutdown ! interface Vlan1 no ip address shutdown ! ! ! ! line con 0 ! line vty 0 4 login line vty 5 15 login ! ! ! ! end

## Switch 4

Current configuration : 1453 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname 242100Q_TokyoSwitch ! enable password 7 0873181C584955262D0E0205282721 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! interface FastEthernet0/1 switchport port-security maximum 3 ! interface FastEthernet0/2 switchport port-security maximum 3 ! interface FastEthernet0/3 shutdown ! interface FastEthernet0/4 shutdown ! interface FastEthernet0/5 shutdown ! interface FastEthernet0/6 shutdown ! interface FastEthernet0/7 shutdown ! interface FastEthernet0/8 shutdown ! interface FastEthernet0/9 shutdown ! interface FastEthernet0/10 shutdown ! interface FastEthernet0/11 shutdown ! interface FastEthernet0/12 shutdown ! interface FastEthernet0/13 shutdown ! interface FastEthernet0/14 shutdown ! interface FastEthernet0/15 shutdown ! interface FastEthernet0/16 shutdown ! interface FastEthernet0/17 shutdown ! interface FastEthernet0/18 shutdown ! interface FastEthernet0/19 shutdown ! interface FastEthernet0/20 shutdown ! interface FastEthernet0/21 shutdown ! interface FastEthernet0/22 shutdown ! interface FastEthernet0/23 shutdown ! interface FastEthernet0/24 shutdown ! interface GigabitEthernet0/1 shutdown ! interface GigabitEthernet0/2 shutdown ! interface Vlan1 no ip address shutdown ! ! ! ! line con 0 ! line vty 0 4 login line vty 5 15 login ! ! ! ! end